HUB2 A7608E-H Modem Integration Plan

Temco Controls | May 2026

1. Objective

The goal is to integrate the A7608E-H 4G/GNSS modem into the HUB2 project. The ESP32-S3 will control the modem through AT commands and support SIM detection, LTE registration, signal status, cellular data connection, GNSS location, and basic remote communication.

The HUB2 hardware includes ESP32-S3, A7608E-H, W5500 Ethernet, RTL8309N Ethernet switch, SIM interface, LTE antenna, GNSS antenna, and power management. The integration should first focus on stable modem bring-up and status reporting. Full 4G-to-Ethernet routing should be treated as a separate, higher-risk phase.

2. Phase 1 - Hardware Bring-up

Do not start by merging the modem code into the main Temco firmware. First verify the hardware with a small test project or LilyGO-style AT debug example.

1.    Confirm that the ESP32-S3 can be flashed and boots correctly.

2.    Verify the A7608E-H power rail during power-on, network registration, and data transmission.

3.    Verify PWRKEY, RESET, FLIGHTMODE, STATUS, and NETLIGHT control/status signals.

4.    Confirm UART communication between ESP32-S3 and A7608E-H using basic AT / OK test.

5.    Check SIM card detection, SIM power, SIM clock, reset, and data lines.

6.    Check LTE main antenna and GNSS antenna connections.

7.    Check whether the modem USB path can be used for debugging or firmware upgrade.

| Deliverable | Description |
| --- | --- |
| Pin map | Final ESP32-S3 to A7608E-H signal table based on the HUB2 schematic. |
| Power sequence | Confirmed modem power-on, reset, and power-off timing. |
| AT log | Basic AT communication log showing modem response. |
| Power test | Voltage/current record during idle, registration, and data transmission. |
| Issue list | Any hardware issues found during bring-up. |

3. Phase 2 - A7608 Driver

Create a standalone A7608 driver. AT command handling should be kept in one module instead of being scattered across the application code.

Modem power on / power off

Modem reset

AT command send / receive

Timeout and retry handling

SIM status detection

LTE registration status

Signal strength and operator information

IP attach status and cellular IP address

GNSS enable, disable, and location readout

Suggested initial AT commands:

AT

ATI

CPIN?

CSQ

CREG?

CEREG?

CGATT?

CGDCONT?

CGPADDR

NETOPEN / IPCLOSE / IPADDR

GNSS related commands

| Deliverable | Description |
| --- | --- |
| a7608.c / a7608.h | Reusable modem driver files. |
| AT framework | Blocking or state-machine based AT command handling with timeout/retry. |
| Modem state machine | States for off, booting, SIM ready, registered, attached, connected, and error. |
| Debug output | Clear log messages for bring-up and field troubleshooting. |

4. Phase 3 - Cellular Data Connection

At this stage the target should be "connect out" first, not full router behavior. This keeps the first prototype realistic and easier to debug.

1.    Establish cellular data connection through AT commands.

2.    Test TCP client, MQTT, or HTTP upload using modem commands.

3.    Send basic device status through the cellular link.

4.    Add reconnect logic for LTE dropouts.

5.    Add modem reset recovery if the modem stops responding.

If the product only needs remote status upload or cloud connection, this phase is enough for the first prototype.

If the target is Ethernet devices using HUB2 as a 4G router, then PPP, NAT, and IP forwarding are required. That is a much larger software task and should be defined separately.

5. Phase 4 - Temco Firmware Integration

After the modem driver is stable, integrate it into the Temco ESP32 firmware as a new board/project type, such as a HUB2 project type or a 4G Hub project type.

1.    Add HUB2 board configuration and pin definitions.

2.    Initialize W5500 Ethernet interface.

3.    Initialize A7608 modem driver.

4.    Add modem status into the system status structure.

5.    Expose modem status through Modbus registers.

6.    Optionally expose modem status through BACnet objects.

7.    Make the status readable from T3000.

| Status Item | Purpose |
| --- | --- |
| SIM inserted / ready | Shows whether the SIM card is usable. |
| LTE registered | Shows whether the modem has registered on the network. |
| CSQ / RSSI | Shows cellular signal quality. |
| Operator name | Shows the current mobile carrier. |
| Cellular IP address | Shows modem data connection status. |
| GNSS latitude / longitude | Shows location if GNSS is enabled. |
| Modem uptime | Helps identify modem resets. |
| Modem reset count | Helps diagnose instability. |
| Last error code | Useful for support and field troubleshooting. |
| Ethernet link status | Shows W5500/Ethernet status. |
| Battery / input voltage | Helps diagnose power issues. |

6. Phase 5 - W5500 and Ethernet Switch Verification

The W5500 should be treated as the ESP32 Ethernet interface. The RTL8309N can initially be treated as an unmanaged hardware Ethernet switch. No RTL8309N management driver is needed unless we need VLAN, port isolation, or per-port status later.

1.    ESP32 obtains static IP or DHCP through W5500.

2.    T3000 can discover the HUB2 board.

3.    Modbus TCP access works through Ethernet.

4.    BACnet/IP discovery works through Ethernet.

5.    Multiple Ethernet ports can pass traffic normally.

6.    LTE modem and Ethernet functions can run at the same time without instability.

7. Phase 6 - Stability Test

Run continuously for 24 to 72 hours.

Test LTE reconnect under weak signal condition.

Test SIM card removal and re-insertion.

Test LTE/GNSS antenna removal and recovery behavior.

Test Ethernet cable plug/unplug.

Test USB power and battery power behavior.

Check whether ESP32 resets during modem high-current transmission.

Check whether the system recovers after modem reset.

Verify T3000 scan, Modbus TCP, and BACnet/IP stability.

8. Main Risks

1.    The A7608E-H modem has high peak current demand. If the power design is weak, the modem or ESP32 may reset.

2.    The copied LilyGO-style power switching may not be seamless. USB/battery switching may cause reset behavior.

3.    UART AT mode is good for status upload and simple TCP/MQTT/HTTP, but it is not suitable for a high-speed 4G router.

4.    True 4G-to-Ethernet routing requires PPP, NAT, and IP forwarding, which adds major software complexity.

5.    LTE and GNSS performance depends heavily on antenna layout, grounding, and matching.

6.    Running W5500, modem, Wi-Fi, BACnet, and Modbus together may require careful ESP32 memory and task-priority management.

9. Suggested Schedule

| Week | Task | Expected Result |
| --- | --- | --- |
| Week 1 | Hardware bring-up | ESP32 boot, A7608 power-on, UART AT, SIM detection, LTE registration, basic power test. |
| Week 2 | A7608 driver | AT framework, modem state machine, SIM/LTE/RSSI/GNSS status readout. |
| Week 3 | Cellular data | LTE data connection, TCP/MQTT/HTTP test, reconnect and recovery logic. |
| Week 4 | Temco integration | HUB2 project config, W5500 integration, A7608 status, Modbus/BACnet status points. |
| Week 5 | System test | Long-run test, weak LTE signal test, Ethernet test, T3000, BACnet/IP, and Modbus TCP verification. |

10. Recommended Direction

The modem integration should be done in two steps. First, integrate the A7608E-H as a cellular communication module with AT control, LTE connection, GNSS, and status reporting. This is achievable and suitable for the first prototype.

After the first prototype is stable, we can decide whether HUB2 also needs true 4G-to-Ethernet routing. That second direction requires PPP, NAT, and IP forwarding, so it should be planned as a separate phase with more time and risk control.